## contribution

- physics-aware robustness evaluation of 2D tokamak equilibrium reconstruction, assessed on boundary, axis and X-point accuracy rather than pixel error.

- A sensor-criticality analysis grounded in physics, which identifies safety-critical and redundant diagnostics and a minimal sensor set.

- A failure-aware, physics-constrained reconstruction model.

- An adaptive mitigation pipeline with calibrated confidence that reduces false wall-proximity alarms in real time.

Baseline -

- TokaMark CNN (MLP backbone) and CNN+LSTM on Task 1-3, on the official random and temporal splits

- Gupta's six failure scenarios with zero-, mean- and forward-fill imputation, plus the Robustness Score

- NRMSE, MAE and RMSE as metrics

## Novel modules

### Physics-aware evaluation layer

- Extract the plasma boundary, magnetic axis and X-point from the predicted ψ.

- Report:

  - boundary error and plasma

  - axis shift

  - X-point detection rate

  - elongation and triangularity error

  - false wall-proximity alarm rate

both prior papers use only pixel- or signal-level NRMSE.

### Failure-aware reconstruction model

- A model that takes a missingness mask as input, trained with structured failure augmentation

- architecture: one token per sensor in a Transformer, so missing sensors are simply dropped.

- Add a flux-loop consistency loss: predicted ψ at each loop's position must match the measured flux.

### Physics-grounded sensor criticality analysis

- Rank individual sensors and groups by physics impact (boundary and X-point shift), not by NRMSE.

- Show spatial error maps per failure and compare them with each sensor's location.

- Use Shapley values over sensors to expose redundancy and interactions; search for the worst set of k failures.

- Compare the data-driven ranking with Green's-function sensitivity.

- End with a design answer: the minimum sensor set that keeps boundary error below a tolerance.

### Adaptive failure mitigation with confidence output

- Detect faults, including stuck rails and drift, using cross-sensor and flux-loop residuals.

- Isolate the faulty sensor and mask it.

- Choose a mitigation by duration and criticality: forward-fill for short gaps, a virtual sensor from redundant neighbours for long gaps, and routing to the failure-aware model or physics fallback for critical losses. N3's results set these rules.

- Output calibrated uncertainty (conformal intervals on boundary position) and a confidence flag for the controller.

## Result

Each row adds one module, so the table shows what each contributes. Every cell reports boundary error (cm) alongside NRMSE.

| Configuration | Clean | Channel loss | Group loss | Temporal gap | Pre-disruption | Latency (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| TokaMark CNN + zero-fill (baseline) |  |  |  |  |  |  |
| + best fixed imputation (Gupta-style) |  |  |  |  |  |  |
| + failure-aware training |  |  |  |  |  |  |
| + flux-loop consistency loss |  |  |  |  |  |  |
| + adaptive mitigation |  |  |  |  |  |  |
| Oracle mitigation |  |  |  |  |  |  |

Required

- 3–5 seeds, reported as mean ± std

- Fair tuning for every model